DZS 065 2026

For Safety and Quality Assurance

Zambian Standard GLYCERINE FOR COSMETICS - Specification

ZAMBIA BUREAU OF STANDARDS

Amendments issued since publications

| Amdt No. | Date | Text affected |
| --- | --- | --- |

COPYRIGHT PROTECTED DOCUMENT

© ZABS 2026

All rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from Zambia Bureau of Standards

DATE OF PUBLICATION

This Zambian Standard has been published under the authority of the Bureau on XXXXXX.

ZAMBIA BUREAU OF STANDARDS

The Zambia Bureau of Standards is a statutory organization established by the Standards Act No.4 of 20217, for the preparation and promulgation of Zambian Standards.

REVISION OF ZAMBIAN STANDARDS

Zambian Standards are revised, when necessary, by the issue either of amendments or of revised editions.  It is important that users of Zambian Standards should ascertain that they are in possession of the latest amendments or edition.

CONTRACT REQUIREMENTS

A Zambian Standard does not purport to include all the necessary provisions of a contract.  Users of Zambian Standards are responsible for their correct application.

TECHNICAL COMMITTEE RESPONSIBLE

The preparation of this Zambian Standard was undertaken by Technical committee on Cosmetics products which the following organization were represented:

Zambia Association of Consumers

Competition and Consumer Protection Commission

Zambia Compulsory Standards Agency

Zambia Bureau of Standards

Glycerine for cosmetic industry — Specification

## Scope

This Zambian standard specifies requirements, sampling and test methods for glycerin for cosmetic       industry.

## Normative references

The following referenced documents referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ZS 185- 1 Glossary of terms relating to the cosmetic industry

ZS 185-2, Oils for cosmetic industry — Methods of test — Part 2: Determination of moisture content ZS 185-7, Oils for cosmetic industry — Methods of test — Part 7: Determination of specific gravity

ZS 185 -15, Cosmetics — Analytical methods — Part 15: Determination of ash content

ZS 185-16, Oils for cosmetic industry — Methods of test — Part 16: Determination of heavy metal content

ZS 954   (all parts), Cosmetics and cosmetic products

ISO 24153, Random sampling and randomisation procedures

## Terms and definitions

For the purposes of this document, the terms and definitions given in EAS 846 and the following apply. ISO and IEC maintain terminological databases for use in standardization at the following addresses:

— ISO Online browsing platform: available at http://www.iso.org/obp

specific gravity

ratio of the density of a material to the density of a standard material such as water (for liquids) or air (for gases)

## Requirements

  - General requirements

    - The raw material shall comply with the requirements of all parts of ZS 954.

    - The product shall be:

      - a clear syrupy liquid containing essentially of glycerol and having a sweet warm taste;

      - free from free from sediment, suspension and other foreign matter; and

      - miscible in all proportions with water and with ethyl alcohol (90 % v/v) and shall not be soluble in diethyl ether, chloroform and fatty oils

    - The product shall be produced, prepared and handled in accordance with ISO 22716.

  - Specific quality requirements

The product shall comply with the requirements given in Table 1 when tested in accordance with the test methods prescribed therein.

Table 1 — Specific quality requirements for glycerine for cosmetic industry

| Characteristic | Requirement | Test method |
| --- | --- | --- |
| Glycerol, % by mass, min. | 98 | Annex A |
| Moisture content, % by mass, max. | 2 | ZS 185- 2 |
| Specific gravity, at 25 °C | 1.2490 - 1.2636 | ZS 185 -7 |
| Ash, % by mass, max. | 0.01 | ZS 847-15 |
| Alkalinity (as Na2O), % by mass, max. | 0.01 | Annex B |
| Chlorides (as Cl), mg/l, max. | 10 | Annex C |
| Sulphates (as SO4), mg/l, max. | 10 | Annex D |
| Fatty acids and esters (as Na2O), % by mass, max. | 0.06 | Annex E |

## Heavy metal requirements

Glycerine shall comply with the heavy metal requirements given in Table 2 when tested in accordance with the test methods specified therein.

Table 2 — Specific quality requirements for glycerine for cosmetic use

| Heavy metals, | Limit mg/l, max. | Test method |
| --- | --- | --- |
| Lead | 5 | ZS 185-16 |
| Arsenic | 2 |  |
| Mercury | 2 |  |
| Cadmium | 2 |  |

## Packaging

The container (including the closure) in which the glycerine is packaged shall not react chemically with the glycerine and shall be strong enough to protect the product adequately during handling, transportation and storage.

## Labelling

In addition to the labelling requirements given i, the package shall be legibly marked with the following information:

- manufacturer's name and physical address;

- product name as “Glycerine”;

- net content of the product when packed;

- batch number;

- country of origin;

- instructions for use;

- date of manufacture and expiry; and

- precaution/warning, if any.

## Sampling

Representative samples of the product shall be drawn randomly for test in accordance with ISO 24153 from the market, factory or elsewhere.

# Annex A

(normative)

# Determination of glycerol content

## Apparatus

    - Burette

    - Pipette

    - Conical flasks with glass stopper, 500-ml capacity

    - Weighing pipette/bottle

## Reagents

    - Sodium hydroxide solution, 0.1 N, accurately standardized

    - Sodium hydroxide, 0.05 N

    - Phenol red indicator, 0.05 % m/v. Dissolve 0.05 g of phenol red in 10 ml of ethyl alcohol and make up to100 ml with water.

    - Dilute sulphuric acid, 0.2 N

    - Sodium metaperiodate solution. Dissolve 70 g of sodium metaperiodate in one litre of water containing 10 ml of 1 N sulphuric acid and store the solution in an amber coloured bottle in the dark.Sodium metaperiodate shall be a white crystalline powder containing not less than 98 percent sodium metaperiodate.

    - Ethylene glycol, neutral and free from glycerol

    - Standard glycerol

## Procedure

    - Using a weighing pipette or bottle, transfer into a conical flask a well-mixed and accurately weighed sample.

    - Add 100 ml of carbon dioxide-free water and three drops of phenol red indicator solution and acidify with sulphuric acid solution to definite yellow colour. Heat to boiling and cool to room temperature.

    - Adjust the pH value to 8.0 ± 0.1 using 0.05 N sodium hydroxide solution dropwise until the colour changes to just pink. If the colour of the original solution interferes with the detection of colour change of the indicator, use a pH meter.

    - Pipette accurately 50 ml of sodium metaperiodate solution, replace the glass stopper, mix by swirling gently and allow the flask to stand in the dark for 30 min.

    - Wash down the sides of the flask with water, add 5 ml of ethylene glycol, replace the stopper, swirl gently and allow the flask to stand in the dark for further 20 min and titrate with standardized 0.05 N sodium hydroxide solution.

    - Carry out a blank test simultaneously under similar test conditions.

  - Calculation

The glycerol content, expressed as percent by mass, shall be calculated as follows:

9.209(S  B ) N M

where

S	is the volume, in millilitres, of standard sodium hydroxide solution required for the sample; B	is the volume, in millilitres, of standard sodium hydroxide solution required for the blank; N	is the normality of standard sodium hydroxide solution; and

M	is the mass, in grams, of the sample taken for test.

Each millilitre of 0.05 N sodium hydroxide is equivalent to 9.210 mg of C3H8O3.

# Annex B

(normative)

# Test for alkalinity

  - Procedure

    - Weigh accurately about 100 g of a well-mixed sample into 500-ml conical flask and add 150 ml of carbon dioxide-free water.

    - Titrate with standardized 0.1 N hydrochloric acid using phenolphthalein indicator.

  - Reagents

    - Standardized hydrochloric acid, 0.1 N

    - Phenolphthalein indicator solution

  - Apparatus

    - Conical flask, 500-ml capacity

    - Burette

B.4 Calculation

The alkalinity (Na2O), expressed as percent by mass, shall be calculated as follows:

3.1  A N M

where

A	is the volume, in millilitres, of standardized 0.1 N hydrochloric acid;

N	is the normality of standardized 0.1 N hydrochloric acid; and

M	is the mass in grams of sample taken.

Annex C

(normative)

# Determination of chlorides (Volumetric method)

C.4 Calculation

The chlorides (as Cl) content, expressed as parts per million, shall be calculated as follows:

35500(B S ) N M

where

C.1 Apparatus

Conical flasks, 250-ml capacity

  - Reagents

    - Nitric acid solution, 35 % v/v

    - Nitrobenzene

    - Silver nitrate solution, 0.05 N accurately standardized

    - Ammonium thiocyanate solution, 0.05 N accurately standardized

    - Ferric ammonium sulphate solution, saturated, aqueous

  - Procedure

    - Weigh accurately about 20 g of the sample in a 250-ml conical flask and add about 100 ml of water.

    - To this solution, add 5 ml of nitric acid, a few drops of nitrobenzene and 5 ml of standard 0.05 N silver nitrate solution.

    - Swirl and titrate with standardized 0.05 N ammonium thiocyanate solution using 1 ml of ferric ammonium sulphate solution as indicator. Carry out a blank test using the same reagents.

B	is the volume, in millilitres, of standard ammonium thiocyanate solution required for the blank; S	is the volume, in millilitres, of standard ammonium thiocyanate solution required for the sample; N	is the normality of standard ammonium thiocyanate solution; and

M	is the mass, in grams, of the material taken for the test.

# Annex D

(normative)

# Determination of sulphates

## Apparatus

    - Nessler cylinders, 25-ml capacity

    - Pipette, 1-ml capacity with 0.01-ml graduations

## Reagents

    - Hydrochloric acid, 1:1 solution of hydrochloric acid in water

    - Barium chloride solution, 10 % (m/v), aqueous

    - Standard sulphate solution. Dissolve 1.479 g of anhydrous sodium sulphate in water and make up to one litre in a volumetric flask. Dilute this stock solution 10 times immediately before use. One millilitre of this solution contains 0.000 1 g of sulphate (SO4).

## Procedure

    - Weigh accurately about 10 g of the sample into a Nessler cylinder, add 1 ml of hydrochloric acid and 2 ml of barium chloride solution, and make up to 25 ml with water. Allow to stand for 30 min. A dull white turbidity is produced if sulphates are present.

    - Simultaneously, introduce 0.5 ml, 0.75 ml, 1.0 ml and 1.25 ml of standard sulphate solution into four separate Nessler cylinders. In each, add 1 ml of hydrochloric acid and 2 ml of barium chloride solution, and make up to 25 ml with water. Allow to stand for 30 min.

    - Note which one of these four solutions has a turbidity most closely matching that produced by the test material (see D.3.1). Repeat the test and note the volume of standard sulphate solution required to produce a turbidity exactly matching that of the test sample.

## Calculation

The sulphates (as SO4) content, expressed as parts per million, shall be calculated as follows:

100V M

where

V	is the volume, in millilitres, of standard sulphate solution used; and

M	is the mass, in grams, of the material taken for the test.

# Annex E

(normative)

# Determination of fatty acids and esters.

## Apparatus

    - Round-bottomed flask, 500-ml capacity

    - pH meter

## Reagents

    - Sodium hydroxide solution, 0.25 N accurately standardized

    - Sulphuric acid, 0 25 N accurately standardized

    - Phenolphthalein indicator solution

## Procedure

    - Weigh accurately about 50 g of the sample into a 500-ml round-bottomed flask. Add 100 ml of hot, carbon dioxide-free water and 1 ml of phenolphthalein indicator solution. If the solution is alkaline, neutralize it with sulphuric acid

    - Add exactly 15.0 ml of standard sodium hydroxide solution, connect the flask to a reflux condenser and heat to boiling. Boil for 5 min, allow to cool slightly and wash down the condenser with a little water. Disconnect the flask, close it with a stopper carrying a sodalime tube, and cool.

    - Titrate with standardized 0.25 N sulphuric acid and perform a blank determination.

## Calculation

The fatty acids and esters (as Na2O) content, expressed as percent by mass, shall be calculated as follows:

3.1( B  S)N M

where

B	is the volume, in millilitres, of standard sulphuric acid required for the blank;

S	is the volume, in millilitres, of standard sulphuric acid required for the material;

N	is the normality of standard sulphuric acid; andM is the mass, in grams, of the material taken for the test.