# DZS1324: 2026

For Safety and Quality Assurance

## Zambian Standard

# DEODORANTS AND ANTIPERSIRANTS - Specification

## ZAMBIA BUREAU OF STANDARDS

Amendments issued since publications

| Amdt No. | Date | Text affected |
| --- | --- | --- |

COPYRIGHT PROTECTED DOCUMENT

© ZABS 1990

All rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from Zambia Bureau of Standards

## DATE OF PUBLICATION

This Zambian Standard has been published under the authority of the Bureau on 1990-03-31.

## ZAMBIA BUREAU OF STANDARDS

The Zambia Bureau of Standards is the Statutory National Standards Body for Zambia established under an act of Parliament, the Standards Act, No. 4 of 2017 of the Laws of Zambia for the preparation and promulgation of Zambian standards

## REVISION OF ZAMBIAN STANDARDS

Zambian Standards are revised, when necessary, by the issue either of amendments or of revised editions.  It is important that users of Zambian Standards should ascertain that they are in possession of the latest amendments or edition.

## CONTRACT REQUIREMENTS

A Zambian Standard does not purport to include all the necessary provisions of a contract.  Users of Zambian Standards are responsible for their correct application.

## TECHNICAL COMMITTEE RESPONSIBLE

The preparation of this Zambian Standard was undertaken by Cosmetics and Hygienic products Technical Committee (CHD/1) on which the following organization were represented:

Ministry of Health (Food and Drugs control Laboratories)

Tapera Industries

Zambia Bureau of Standards

_____________________________________________________________________

##### ZAMBIA BUREAU OF STANDARDS, P.O. BOX 50259, LUSAKA

Contents	Page

Foreword	iv

- Scope	1

- Normative references	1

- Terms and definitions	1

- Requirements	2

  - Ingredients	2

  - General requirements	2

  - Specific requirements	2

- Microbiological limits	3

- Packaging	3

  - General	3

  - Roll-ball construction	3

- Labelling	4

- Sampling	4

Annex A (normative)  Determination of stability of smell	5

  - Apparatus	5

  - Procedure	5

  - Result	5

Annex B (normative) Determination of non-volatile matter	6

  - Apparatus	6

  - Procedure	6

  - Calculation	6

Annex C (normative)  Determination of delivery rate of dispenser	7

  - Material and apparatus	7

  - Procedure	7

Annex D (normative) Determination of propellant composition	8

  - Procedure	8

  - Results	9

Annex E (normative) Net weight delivery	10

Annex F (normative) Spray test	11

Annex G (normative) Determination of valve leakage	12

Annex H (normative) Testing of filled aerosol containers	13

  - Procedure	13

  - Interpretation of results	13

Bibliography	14

# Foreword

This National Standard has been prepared in accordance with the procedures of ZABS. All users should ensure that they have the latest edition of this publication as normative documents are revised from time to time.

No liability shall attach to ZABS or its Director, employees, servants or agents including individual experts and members of its Technical Committees for any personal injury, property damage or other damages of any nature whatsoever, whether direct or indirect, or for costs (Including legal fees) and expenses arising out of the publication, use of, or reliance upon this ZABS publication or any other ZABS publication.

Compliance with a Zambian standard or other normative document does not of itself confer immunity from legal obligations

# Introduction

Use of deodorants and antiperspirants is on the increase. Antiperspirants are used to prevent sweating whilst deodorants are used to mask body odour. The safety of the consumer and quality of the product need to be taken into consideration by laying down the requirements for the product.

Deodorants and antiperspirants are packaged as roll-ons, aerosols, squeeze, stick products and any other permitted packaging.

DRAFT ZAMBIAN STANDRD                                             DZS 1324 2026

# Deodorants and antiperspirants — Specification

## Scope

This Draft Zambian Standard specifies the requirements, sampling and test methods for deodorants and antiperspirants.

## Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ZS 954  (all parts), Cosmetics and cosmetic products

ZS 185 16 , Cosmetics — Analytical methods — Part 16: Determination of heavy metal content

ZS 185 17, Cosmetics — Analytical methods — Part 17: Physio-chemical tests

ISO 7010, Graphical symbols — Safety colours and safety signs — Registered safety signs

ISO 22716, Cosmetics — Good Manufacturing Practices (GMP) — Guidelines on Good Manufacturing Practices

ISO 24153, Random sampling and randomisation procedures

## Terms and definitions

For the purposes of this document, the terms and definitions given in EAS 846 and the following apply. ISO and IEC maintain terminological databases for use in standardization at the following addresses:

- IEC Electropedia: available at http://www.electropedia.org/

- ISO Online browsing platform: available at http://www.iso.org/obp

##### 3.1

antiperspirant

preparation for preventing the flow of sweat

##### 3.2

deodorant

substance applied to the body to mask the smell of perspiration. May be packed as sticks, pads, dabber units, aerosols, roll-ons, pump sprays, squeeze bottles and creams

##### 3.3

roll-ball

spherically shaped object, with the capacity to roll in all directions. It is put at the opening of a roll-on container and serves the role of closing the container as well as dispensing the contents, when rolled on the skin.

## Requirements

#### Ingredients

All ingredients used including dyes, pigments and colour shall conform to all parts of ZS 934.

#### General requirements

    - The preparation shall be of uniform colour and shall be free from visible impurities.

    - Deodorants and antiperspirants shall not be harmful to the user when used as intended by the manufacturer.

    - Deodorants and antiperspirants shall be produced, prepared and handled in accordance with ISO 22716.

#### Specific requirements

    - Deodorants and antiperspirants shall also comply with the requirements given in Table 1 when tested in accordance with the test methods specified therein.

##### Table 1 — Specific requirements for deodorants and antiperspirants

| S/NO | Characteristic |  | Requirement | Test method |
| --- | --- | --- | --- | --- |
| i. | Stability of smell |  | To pass test | Annex A |
| ii. | pH neata |  | 3 – 7 | ZS 185 17 |
| iii. | Non-volatile matter, % m/m, min.b | Aerosols | 1.0 | Annex B |
|  |  | Non-aerosols | 5.0 |  |
| a, b These tests do not apply to stick products. |  |  |  |  |

    - Deodorants and antiperspirants packed in aerosol containers shall in addition comply with the requirements given in Table 2 when tested in accordance with the test methods specified therein.

##### Table 2 — Specific requirements for deodorants and antiperspirants packed in aerosol containers

| S/NO | Characteristic | Requirement | Test method |
| --- | --- | --- | --- |
| i. | Delivery rate g/s, min. | 0.01 | Annex C |
| ii. | Chlorofluorocarbons (CFCs) | Absent | Annex D |
| iii. | Net weight delivery m/m, %, min. | 95 | Annex E |
| iv. | Spray test | To pass test | Annex F |
| v. | Valve leakage, g/year, max. | 5 | Annex G |
| vi. | General leakage | To pass test | Annex H |

- Heavy metal contaminants

Deodorants and antiperspirants shall comply with the limits for heavy metal contaminants given in Table 3 when tested in accordance with the test methods specified therein.

##### Table 3 — Heavy metal limits for deodorants and antiperspirants

| S/NO | Heavy metal | Maximum limit a mg/kg | Test method |
| --- | --- | --- | --- |
| i. | Lead (as Pb) | 10.0 | ZS 185-16 |
| ii. | Arsenic (as As) | 2.0 |  |
| iii. | Mercury (as Hg) | 1.0 |  |
| iv | Cadmium | 2.0 |  |
| a The total amount of heavy metals as lead, mercury and arsenic, in combination in the finished product shall not exceed 10 mg/kg. |  |  |  |

6	Microbiological limits

Deodorants and antiperspirants shall comply with the microbiological limits given in Table 4 when tested in accordance with the test methods specified therein.

Table 4 — Microbiological limits for deodorants and antiperspirants

| S/NO | Microorganism | Limit | Test method |
| --- | --- | --- | --- |
| i. | Total viable count for aerobic mesophyllic microorganisms, CFU/g or CFU/ml, max. | 1 000 | ISO 21149 |
| ii. | Pseudomonas aeruginosa | Not detectable in 1 ml or 1 g of cosmetic product | ISO 22717 |
| iii. | Staphylococcus aureus |  | ISO 22718 |
| iv. | Candida albicans |  | ISO 18416 |

- Packaging

  - General

Deodorants and antiperspirants shall be packaged as roll-ons, aerosols, squeeze, stick products and in any other suitable containers that shall protect the contents and shall not cause any contamination or react with the product.

  - Roll-ball construction

If the container is fitted with a roll ball, the roll ball shall be

- made of plastic material,

- fitting on the container such that on holding the container upside down the contents shall not pour out, and

- free rolling, leaving a thin layer of the contents on the skin during dispensation.

## Labelling

  - In addition to the labelling requirements, the package shall be legibly and indelibly marked with the product name as “Deodorant” or “Antiperspirant”;

  - In addition, the following warning shall be labelled on all products containing Aluminium zirconium chloride hydroxide complexes and/or the Aluminium zirconium chloride hydroxide glycine complexes: CAUTION ‘Do not apply to irritated or damaged skin.’

8.3	The product shall also be labelled with appropriate safety symbols as specified in ISO 7010.

9	Sampling

Sampling shall be done in accordance with ISO 24153.

# Annex A

(normative)

# Determination of stability of smell

A.3 Result

The product shall be taken to have passed the test if, after 12 h, the smell of the sample can clearly be picked up.

A.2 Procedure

Put some pieces of bleached gauze which have been pre-washed in hot water without soap and dried into a porcelain cup and pour 1.5 ml of the sample into this cup. After the gauze gets soaked, take it out with the help of pincers. Without squeezing it, dry it in a premise having temperature 37 °C ± 2 °C and humidity of 65

% ± 5 % for 12 h.

  - Apparatus

    - Porcelain cup

    - Pincers

    - Bleached gauze, ten pieces of dimension 5 cm x 10 cm

    - Thermometer

    - Hygrometer

Annex B

(normative)

# Determination of non-volatile matter

where

M2   is the mass, in grams, of the dish and dried material;

M1   is the mass, in grams, of the dry and empty dish; and

M	is the mass, in grams, of the material taken.

× 100

=

B.3 Calculation

The non-volatile matter content, expressed as percent by mass, shall be calculated using the formula below:

B.2 Procedure

Chill the sample to 2 ºC ±1 ºC for 30 minutes. Weigh accurately 1 g ± 0.2 g of the sample in the dish and place it in an oven at 105 °C ± 2 °C for 1 h. Cool to room temperature in a desiccator and weigh the dish. Repeat the process to bring it to constant mass.

  - Apparatus

    - Moisture dish

    - Oven

    - Analytical balance

    - Desiccator

Annex C

(normative)

# Determination of delivery rate of dispenser

## Material and apparatus

##### Any suitable timing device

    - Analytical balance with an accuracy to 0.01 g and capacity greater than 500 g

    - Pair of gloves made of cloth or fabric or towel for handling dispensers during test

    - Pair of tongs for removing dispensers from water bath

    - Water bath set at 26 °C ± 0.3 °C, thermostatically controlled

## Procedure

    - Hold a dispenser upright, spray for two seconds to fill the induction tube. Then weigh the dispenser.

    - Submerge the dispenser into the water bath for 15 min using tongs, remove the dispenser from the bath and immediately dry the container with a towel. Spray the dispenser in one continuous burst for 10 s. Reweigh the dispenser.

    - Repeat the procedure and take an average of three tests. The difference between the maximum and minimum delivery rates shall not exceed 0.2 g/s.

##### Calculation

The delivery rate, expressed in grams per second, shall be calculated using the formula below:

=

where

M1   is the initial weight, in grams, of the dispenser;

M2   is the final weight, in grams, of the dispenser; and

N	is the time in seconds.

# Annex D

(normative)

# Determination of propellant composition

## Procedure

    - The analysis of the propellant mixture in most aerosol is carried out conveniently by gas chromatography. For sampling, a hypodermic needle is fitted to the valve of the aerosol can and approximately 0.5 g of the propellant is injected into the heavy duty centrifuge tube closed with a serum cap, containing about 8 ml of benzene. After mixing, 5-μl samples are taken out from this tube with a microliter syringe and injected into the gas chromatograph.

    - Two 4 572 mm × 76.2 mm OD columns operated at 40 °C are recommended for the analysis containing 20 % weight hexadecane and diethylhexyl sebacate respectively on silanized chromosorb W60/S0 mesh.

The first column should be used mainly for initial screening and the second column for the confirmation and determination of the identified propellants.

Table D.1 lists the relative retention data of the most widely used propellant together with some other fluorinate hydrocarbons and benzene used as the solvent in the two columns.

Table D.1 — Relative retention data of propellants

| Chemical name | Stationary phase diethylhexyl sebacate | Stationary phase hexadecane |
| --- | --- | --- |
| Octafluorocylobutane | 0.214 | 0.122 |
| 1-chloro-1,2,2-trifluoroethylene | 0.268 | 0.196 |
| Propane | 0.275 | 0.22 |
| 1,2-difluoroethane | 0.289 | 0.141 |
| Dichlorodifluoromethane | 0.296 | 0.220 |
| 1,2-dichloro-1,1.2,2-tetrafluoromethane | 0.345 | 0.290 |
| Isobutane | 0.366 | 0.378 |
| Monochlorodiflouoromethane | 0.368 | 0.152 |
| 1-chloro-1,1-difluoroethane | 0.402 | 0.236 |
| n- butane | 0.449 | 0.527 |
| Vinylchloride | 0.529 | 0.353 |
| Trichlorofluoroethane | 1.000 | 1.000 |
| 1,1,2-trichloro-1,2,2-tetrafluoroethane | 1.254 | 1.342 |
| Dichloromonofluoroethane | 1.354 | 0.515 |
| 1,2-dibromo-1,1,2,2-tetrafluoroethane | 1.634 | 1.363 |
| Methylene chloride | 2.565 | 1.070 |
| Benzene | 6.786 | 5.661 |

## Results

The sample shall be considered as having failed the test if it contains any of the CFCs in Table D.1.

# Annex E

(normative)

# Net weight delivery

  - For the determination of the net weight delivery, a random sample of at least three packages is selected. After the removal of any dust cover or caps not required for dispensing the product, the gross weight of each package is determined and after shaking for 15 s, the content of the lightest container is drained by holding the valve wide open. The exhausted container is weighed .The result is called wet-tare weight and is equal to the weight of the container plus any product remaining after draining.

  - Consequently, the regeneration allowance is determined and subtracted from the wet-tare weight to obtain the corrected wet-tare weight. The regeneration allowance is defined as the difference between the weight of the product which would be delivered through normal usage and the weight of the product delivered by the present accelerated procedure. It is calculated by multiplying the label weight of the container by 0.02 g and rounding the result to the next lowest gram.

  - By subtracting the corrected tare weight from the gross weight, the adjusted net weight of the package is obtained. If this is greater than 95 % of the label weight the lot is assumed to be satisfactory. However, if it is less than 95 % of the label weight, the lot is rejected.

# Annex F

(normative)

# Spray test

Hand-shake the sample container for 15 s. Spray for 10 s. The sample shall be deemed to have passed the

test if defects such as “streamers” (solid or nearly solid stream), “droppers” (dripping valve) or no spraying are absent

# Annex G

(normative)

# Determination of valve leakage

Select three aerosol containers, and record the date and time to the nearest half-hour. Weigh each container to the nearest milligram, and record the weight in milligram of each, as W1. Place the containers in an upright position and retain at room temperature for not less than three days. Weigh each container again, recording the weight in milligram of each, as W2 and recording the date and time to the nearest half hour. Determine the time, T, in hours, during which the containers were under test.

Calculate the leakage rate, in milligrams per year, of each container taken using the formula:

Where plastic-coated glass aerosol containers are tested, dry the containers in a desiccator for 18 h, and set upright in a constant-humidity environment for 24 h prior to determining the initial weight as indicated above. Perform the test under the same humidity conditions. Empty the contents of each container tested by utilizing any safe technique (for example, chill to reduce pressure). Empty the residual contents by first rinsing with suitable solvents, then with a few portions of methanol. Retain the container, the valve and all associated parts as a unit, heat as W3 and determine the net fill weight (W1 - W3) for each container tested.

If the average net fill weight has been determined previously, this value may be used as the net fill weight.

# Annex H

(normative)

# Testing of filled aerosol containers

  - Procedure

    - All filled aerosol containers shall be tested by immersion in a water bath set at 55 °C for 3 min.

    - The container shall be such that the pressure generated within the immersed container reaches not less than 90 % of the pressure generated within the containers at equilibrium at 55 °C.

H.2 Interpretation of results

Any filled aerosol container that shall leak, get distorted or burst as a result of this test shall be considered to have failed the test and shall be discarded.

# Bibliography

EAS 960:2020, Deodorants and antiperspirants — Specification

### DEAS 960: 2024

© EAC 2024 – All rights reserved