ICS: 67.040

DZS 1327 2026

For Safety and Quality Assurance

## Zambian Standard

# HAIR SHAMPOO Synthetic detergent-based Specification

## ZAMBIA BUREAU OF STANDARDS

Amendments issued since publications

| Amdt No. | Date | Text affected |
| --- | --- | --- |

COPYRIGHT PROTECTED DOCUMENT

© ZABS 1990

All rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from Zambia Bureau of Standards

## DATE OF PUBLICATION

This Zambian Standard has been published under the authority of the Bureau on XXXXX.

## ZAMBIA BUREAU OF STANDARDS

The Zambia Bureau of Standards is a statutory organization established by the Standards Act, Cap 416 of 1994, for the preparation and promulgation of Zambian Standards.

## REVISION OF ZAMBIAN STANDARDS

Zambian Standards are revised, when necessary, by the issue either of amendments or of revised editions.  It is important that users of Zambian Standards should ascertain that they are in possession of the latest amendments or edition.

## CONTRACT REQUIREMENTS

A Zambian Standard does not purport to include all the necessary provisions of a contract.  Users of Zambian Standards are responsible for their correct application.

## TECHNICAL COMMITTEE RESPONSIBLE

The preparation of this Zambian Standard was undertaken by Cosmetics and Hygienic products Technical Committee (CHD/1) on which the following organization were represented:

Ministry of Health (Food and Drugs control Laboratories)

Tapera Industries

Zambia Bureau of Standards

_____________________________________________________________________

##### ZAMBIA BUREAU OF STANDARDS, P.O. BOX 50259, LUSAKA

# Hair shampoo Synthetic detergent-based Specification

## Scope

This Draft Zambian Standard prescribes the requirements, sampling and test methods for synthetic detergent-based hair shampoo.

This standard does not cover animal shampoo, soap based hair shampoo and shampoo with medicinal/therapeutic claims.

## Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ZS 954  (all parts), Cosmetics and cosmetic products

ZS 185-3, Cosmetics — Analytical methods —Part 3: Determination of insoluble impurities

ZS 185-16, Cosmetics — Analytical methods — Part 16: Determination of lead, mercury and arsenic content:

ZS 185-17, Cosmetics — Analytical methods — Part 17: Determination of pH

ZS 185-20, Cosmetics — Analytical methods — Part 20: Determination of lather volume (foaming power)

ISO 2870, Surface active agents — Detergents — Determination of anionic-active matter, hydrolysable and non-hydrolysable under acid conditions

ISO 2871-1, Surface active agents — Detergents — Determination of cationic-active matter content — Part 1: High-molecular- mass cationic-active matter

ISO 2871-2, Surface active agents — Detergents — Determination of cationic-active matter content — Part 2: Cationic-active matter of low molecular mass (between 200 and 500)

ISO 18416, Cosmetics —Microbiology —Detection of Candida albicans

ISO 21149, Cosmetics --Microbiology --Enumeration and detection of aerobic mesophilic bacteria

ISO 22717, Cosmetics —Microbiology —Detection of Pseudomonas aeruginosa

ISO 22718, Cosmetics —Microbiology —Detection of Staphylococcus aureus

ISO 24153, Random sampling and randomisation procedures

## Terms and definitions

For the purposes of this document, the terms and definitions given in EAS 846 and the following apply. ISO and IEC maintain terminological databases for use in standardization at the following addresses:

—	ISO Online browsing platform: available at http://www.iso.org/obp

## Classification

Synthetic detergent based hair shampoo shall be classified into four types:

- general purpose shampoo - this shall be for "dry", "oily" or "normal" hair, and shall be indicated so;

- treatment/conditioning/ multi-functional shampoo;

- baby shampoo; and

- neutralizing shampoo.

## Requirements

### Ingredients

    - All ingredients used including dyes, pigments and colours shall comply with EAS 377.

NOTE	A list of ingredients commonly used in the formulation of shampoos is given as information in Annex A.

    - All essential oils/herbs used shall comply with the relevant standards.

    - All active ingredients including detergents, anti-bacterial or agents shall be named. Any further information concerning the active ingredients shall be supplied by the manufacturer on request.

    - For baby shampoos, the active detergent, perfume and all other ingredients shall be of such nature and in such amounts as to leave the final baby product mild in nature. This is due to the sensitive nature of baby skin.

### General requirements

    - The hair shampoo shall be in the form of a liquid, emulsion or paste.

    - The clear/transparent liquid shampoo, when examined visually shall be free from any sediment. If in the form of an emulsion, it shall be homogenous and there shall be no visible signs of the emulsion having broken. Shampoo in the form of a paste shall be free from any agglomerated particles.

    - Hair shampoo may be coloured and perfumed.

    - Hair shampoo shall not be harmful to the user, and shall have no undesirable effect on the natural colour of the hair. (This does not include hair already treated with hair dyes).

    - Hair shampoo shall impart all the effects claimed when used as per the instructions for use.

### Specific requirements

    - Hair shampoo shall comply with the specific requirements given in Table 1 when tested in accordance with the methods prescribed therein

Table 1 — Specific requirements for synthetic detergent based hair shampoo

| S.No. | Characteristic | Requirement |  |  |  | Test method |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | General purpose | Baby | Neutralizing | Multifunctional |  |
| i | Active detergent | 6.0 | 5.0 | 10.0 | 6.0 | ISO 2870 |
|  | matter content, % m/m, min. |  |  |  |  | ISO 2871-1 |
|  |  |  |  |  |  | ISO 2871-2 |
| ii | pH | 4.5 - 9.0 | 5.0 - 9.0 | 4.0 - 5.0 | 4.5 - 9.0 | ZS 185-17 |
| iii | Lather volume for 1 % solution, mL, min. | 100 | 100 | 100 | 100 | ZS 185-20 |
| iv | Matter insoluble in alcohol, % m/m, max. | 2.0 | 2.0 | 2.0 | 2.0 | ZS 185-3 |

    - Baby shampoo shall comply with the specific requirements given in Table 2 when tested in accordance with the methods prescribed therein.

    - Neutralizing shampoo shall comply with the specific requirements given in Table 3 when tested in accordance with the methods prescribed therein.

### Heavy metal contaminants

The hair shampoos shall comply with the limits for heavy metal contaminants in given in Table 4 when tested in accordance with the methods prescribed therein.

Table 4 — Limits for heavy metal contaminants for synthetic detergent-based hair shampoo

| S/N | Heavy metal contaminanta | Maximum limit ,mg/kg | Test method |
| --- | --- | --- | --- |
| i. | Lead | 10 | ZS 185-16 |
| ii. | Arsenic | 2 |  |
| iii. | Mercury | 2 |  |
| a The total amount of heavy metals as lead, mercury and arsenic, in combination, in the finished hair shampoo shall not exceed 10 mg/kg. |  |  |  |

### Microbiological limits

The hair shampoo shall comply with the microbiological limits given in Table 5 when tested in accordance with the methods indicated therein.

Table 5 — Microbiological limits for synthetic detergent-based hair shampoos

|  | Micro-organisms | Limits | Test method |
| --- | --- | --- | --- |
| i | Total viable count for aerobic mesophyllic microorganisms CFU/g or CFU/ml, max. | 100 | ISO 21149 |
| ii | Pseudomonas aeruginosa | Not detected in 1 ml or 1 g of cosmetic product | ISO 22717 |
| iii | Staphylococcus aureus |  | ISO 22718 |
| iv | Candida albicans |  | ISO 18416 |

## Packaging

The hair shampoo shall be packaged in suitable well-sealed containers that shall protect the contents and shall not cause any contamination or react with the products.

## Labelling

Each package shall be legibly and indelibly marked with the following information:

- product name as “Synthetic detergent based hair shampoo” and

- type

## Sampling

Sampling shall be carried in accordance with ISO 24153.

# Annex A

(informative)

# List of ingredients conventionally used in formulation of synthetic detergent based hair shampoo

  - Detergents

    - Sodium or potassium or ethanolamine salts of lauryl sulphonic acid

    - Lauryl ether sulphates

    - Sulphated monoglycetide

    - Sodium alkyl sulpho-acetate

    - Alkyl benzene polyoxyethyl sulphonates

    - Sodium n-lauryl sarcosinate

    - Sodium alpha olefin sulthonates

    - Other synthetic detergents

  - Foam stabilizers

    - Ethanolamides or isopropanolamides of fatty acids

    - Amine oxides

    - Cocobetaines

    - Cocomidopropyl betaines

  - Solubilizing agents

    - Urea

    - Aliphatic alcohols

    - Sodium toluene sulphonate

    - Sodium xylene sulphonate

#### Preservatives

    - Alcohols

    - Formaldehyde Licensed by UNBS to Lydia Nayiga Standards Officer -UNBS

    - Esters of p-hydroxybensoic acid

    - Sorbic acid

    - Imidozolidinyl urea

#### Opacifying agents

    - Higher fatty alcohols

    - Ethylene / propylene glycol stearates

    - Mono and di-stearates of glycerol

    - Zinc, calcium and magnesium salts of fatty acids

    - PEG - distearates 6000

    - Polyacrylates

  - Inorganic salts

    - Sodium chloride

    - Sodium sulphate

    - Sodium phosphate

    - Ammonium sulphate

    - Ammonium phosphate

    - Ammonium chloride

  - Emollients

Lanolin and its derivatives

  - Thickening agents

    - Sodium carboxymethyl cellulose

    - Methyl cellulose

    - Methyl isopropyl cellulose

    - Guar gum

  - Other groups of ingredients

    - Perfumes

    - Dyes

    - Conditioning agents

    - Anti-dandruff agents

    - Anti-bacterial agents

    - Quaternary compounds

    - Vitamins and proteins

    - Vegetable oils and mineral oils

    - Silicones

    - Sunscreens, etc.

# Bibliography

- EAS 842-2: 2017, Hair shampoo — Part 2: Synthetic detergent-based — Specification

- KS 577;2006, Hair shampoo, synthetic detergent based — Specification