DZS 1330 2026 									FIRST EDITION

For Safety and Quality Assurance

## Draft Zambian Standard

# Bath preparations — Synthetic detergent-based foam baths and shower gels

# - Specification

## ZAMBIA BUREAU OF STANDARDS

Amendments issued since publications

| Amdt No. | Date | Text affected |
| --- | --- | --- |

COPYRIGHT PROTECTED DOCUMENT

© ZABS 2026

All rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from Zambia Bureau of Standards

## DATE OF PUBLICATION

This Zambian Standard has been published under the authority of the Bureau on XXXX

## ZAMBIA BUREAU OF STANDARDS

## The Zambia Bureau of Standards is the Statutory National Standards Body for Zambia established under an act of Parliament, the Standards Act, No. 4 of 2017 of the Laws of Zambia for the preparation and promulgation of Zambian standards..

## REVISION OF ZAMBIAN STANDARDS

Zambian Standards are revised, when necessary, by the issue either of amendments or of revised editions.  It is important that users of Zambian Standards should ascertain that they are in possession of the latest amendments or edition.

## CONTRACT REQUIREMENTS

A Zambian Standard does not purport to include all the necessary provisions of a contract.  Users of Zambian Standards are responsible for their correct application.

## TECHNICAL COMMITTEE RESPONSIBLE

The preparation of this Zambian Standard was undertaken by Cosmetics and Hygienic products Technical Committee (CHD/1) on which the following organization were represented:

Ministry of Health (Food and Drugs control Laboratories)

Tapera Industries

Zambia Bureau of Standards

_____________________________________________________________________

##### ZAMBIA BUREAU OF STANDARDS, P.O. BOX 50259, LUSAKA

DRAFT ZAMBIAN STANDARD	DZS 1330 2026

# Bath preparations Synthetic detergent-based foam baths and shower gels— Specification

## Scope

  - This Draft Zambian Standard specifies the requirements, sampling and test methods for synthetic foam baths and shower gels.

  - This standard covers synthetic detergent-based foam baths (also referred to as cream baths), shower gels (also referred to as body wash, cream wash, cream shower, bath shower, and shower shampoo), and other such related products.

  - This standard does not apply to bath salts, bath oils, bath powders, and soap-based bath and shower products.

  - This standard does not apply to medicinal products for which therapeutic claims are made.

## Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ZS 934, (all parts), Cosmetics and cosmetic products

ZS 185-16, Cosmetics — Analytical methods — Part 16: Determination of lead, mercury and arsenic content EAS ZS 185-17, Cosmetics — Analytical methods — Part 17: Determination of pH

ZS 185-20, Cosmetics — Analytical methods — Part 20: Determination of lather volume (foaming power)

ISO 2870, Surface active agents — Detergents — Determination of anionic-active matter, hydrolysable and non-hydrolysable under acid conditions

ISO 2871-1, Surface active agents — Detergents — Determination of cationic-active matter content — Part 1: High-molecular- mass cationic-active matter

ISO 2871-2, Surface active agents — Detergents — Determination of cationic-active matter content — Part 2: Cationic-active matter of low molecular mass (between 200 and 500)

ISO 18416, Cosmetics — Microbiology — Detection of Candida albicans

ISO 21150, Cosmetics — Microbiology — Detection of Escherichia coli

ISO 22717 Cosmetics — Microbiology — Detection of Pseudomonas aeruginosa

ISO 22718, Cosmetics — Microbiology — Detection of Staphylococcus aureus

ISO 24153, Random sampling and randomisation procedures

## Terms and definitions

. ISO and IEC maintain terminological databases for use in standardization at the following addresses:

—	ISO Online browsing platform: available at http://www.iso.org/obp

## Requirements

### General requirements

    - All ingredients used including dyes, pigments and colours shall conform to all parts of EAS 377.

    - The product shall be in the form of a liquid or gel.

    - When examined visually the product shall be:

      - clear or of uniform colour. However, it may be pearlized or have other aesthetic modifications which shall be part of the formulation;

      - homogenous unless a multiphase product is intended, and this should be clearly stated on the label; and

      - free from visible impurities, unless solid particles are added as part of the formulation with a specific functionality, which shall be clearly stated on the label.

    - The foam shall be abundant, and shall be stable in the presence of soap and oil, and within a wide temperature range.

    - The product shall be non-irritant to the skin and mucous membranes when used as recommended by the manufacturer.

    - The product shall not be harmful to the user when used as recommended by the manufacturer.

### Specific requirements

The product shall comply with the specific requirements specified in Table 1 when tested in accordance with the methods prescribed therein.

Table 1 — Specific requirements for synthetic detergent-based foam baths and shower gels

| S/No. | Characteristic |  | Requirement | Test method |
| --- | --- | --- | --- | --- |
| i | Total active detergent matter content, % m/m, min. |  | 6 | ISO 2870 ISO 2871-1 ISO 2871-2 |
| ii | Lather volume for one % solution, mL, min.: |  | after 5 min: 110 | ZS 185-20 |
|  |  |  | after 30 min (foam baths only) : 100 |  |
| iii | pH at 25 °C (neat) | Adult | 4.5 - 9.0 | ZS 185-17 |
|  |  | Baby | 5.0 – 9.0 |  |

### Heavy metal contaminants

The products shall comply with the limits for heavy metal contaminants in accordance with Table 2 when tested in accordance with the methods prescribed therein.

Table 2 — Limits for heavy metal contaminants for synthetic detergent-based foam baths and shower gels

| S/No. | Characteristic | Limita, mg/kg, max | Test method |
| --- | --- | --- | --- |
| i | Lead | 10 | ZS 185- 16 |
| ii | Arsenic | 2 |  |
| iii | Mercury | 2 |  |
| iv | Cadimium | 2 |  |
| a The total amount of heavy metals as lead, mercury and arsenic, in combination, in the finished product should not exceed 20 mg/kg. |  |  |  |

### Microbiological limits

The product shall comply with the microbiological limits given in Table 3 when tested in accordance with the methods indicated therein.

Table 3 — Microbiological limits for synthetic detergent-based foam baths and shower gels

|  | Micro-organisms | Limits | Test method |
| --- | --- | --- | --- |
| I | Total viable count for aerobic mesophyllic microorganisms CFU/g or CFU/ml, max. | 100 | ISO 21149 |
| Ii | Pseudomonas aeruginosa | Not detected in 1 ml or 1 g of cosmetic product | ISO 22717 |
| Iii | Staphylococcus aureus |  | ISO 22718 |
| Iv | Candida albicans |  | ISO 18416 |
| V | Escherichia coli |  | ISO 21150 |

## Packaging

The product shall be packaged in suitable, well-sealed containers that shall protect the product during transportation, handling and storage and shall not cause any contamination or reaction with the product.

## Labelling

In addition, to the labelling requirements given in ZS 036, each package shall be legibly and indelibly marked with the

  - product name as “Foam bath” or “Shower gel”

  - an indication on the use of the product by babies.

## Sampling

Sampling shall be carried out in accordance with ISO 241

DEAS835-1: 2021