|  | DZS 1333 FIRST EDITION |
| --- | --- |

Draft Zambian Standard

Air Freshner aerosols — Specification

ZAMBIA BUREAU OF STANDARDS

AMENDMENTS ISSUED SINCE PUBLICATIONS

| Amdt No. | Date | Text affected |
| --- | --- | --- |

COPYRIGHT PROTECTED DOCUMENT

© ZABS 2026

All rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from Zambia Bureau of Standards

DATE OF PUBLICATION

This Zambian Standard has been published under the authority of the Zambia Bureau of Standards in XXXX.

ZAMBIA BUREAU OF STANDARDS

The Zambia Bureau of Standards is the Statutory National Standards Body for Zambia established under an act of Parliament, the Standards Act, No. 4 of 2017 of the Laws of Zambia for the preparation and promulgation of Zambian standards..

REVISION OF ZAMBIAN STANDARD

Zambian Standards are revised, when necessary, by the issue of either amendments or revised editions. It is important that users of Zambian Standards ascertain that they are in possession of the latest amendments or editions.

CONTRACT REQUIREMENTS

A Zambian Standard does not purport to include all the necessary provisions of a contract. Users of Zambian Standards are responsible for their correct application.

TECHNICAL COMMITTEE RESPONSIBLE

The preparation of this Zambian Standard was entrusted upon the Soaps and Detergents Technical Committee (TC4/2) upon which the following organizations were represented:

Consumer Competition & Protection Commission (CCPC)

Food and Drug Control Laboratory

Tapera Industries

Zambia Bureau of Standards (ZABS)

Zambia Bureau of Standards	Email: zabs@zamnet.zm or infozabs@zamnet.zm

Lechwe House	Website: www.zabs.org.zm

Freedom Way South-end

P.O. Box 50259

Lusaka

FORWORD

This National Standard has been prepared in accordance with the procedures of ZABS. All users should ensure that they have the latest edition of this publication as normative documents are revised from time to time.

No liability shall attach to ZABS or its Director, employees, servants or agents including individual experts and members of its Technical Committees for any personal injury, property damage or other damages of any nature whatsoever, whether direct or indirect, or for costs (Including legal fees) and expenses arising out of the publication, use of, or reliance upon this ZABS publication or any other ZABS publication.

Compliance with a Zambian standard or other normative document does not of itself confer immunity from legal obligations

Contents	Page

Foreword	iii

- Scope	1

- Normative references	1

- Terms and definitions	1

- Requirements.	1

  - General requirements.	1

  - Specific requirements	2

  - Heavy metals requirements	2

- Containers	2

- Packaging	3

- Labelling	3

- Sampling	4

Annex A (normative)  Determination of delivery rate of dispenser	5

  - Material and apparatus	5

A.1.5	Water bath	5

  - Procedure	5

  - Calculation	5

Annex B (normative) Determination of propellant composition	6

  - Procedure	6

  - Results	7

Annex D (normative)  Spray test	9

Procedure	9

Annex E  (normative) Determination of valve leakage	10

Annex F (Normative) Testing of filled aerosol containers	11

  - Procedure	11

  - Interpretation of results	11

Annex G (Normative ) Stability of smell	12

  - Apparatus	12

  - Procedure	12

  - Results	12

Annex H (normative) Flame propagation	13

  - Principle	13

  - Apparatus	13

  - Procedure	14

Annex I (normative) Aerosol containers	15

  - Material	15

  - Construction	15

    - General	15

    - Soldered side seam containers	15

  - Internal finish	15

  - Filling volume	15

Annex J (normative) Determination of cloud temperature	16

  - Procedure	16

  - Results	16

Introduction

Air fresheners are consumer products used in homes, or commercial products used in restrooms, that typically emit fragrance.

Various air fresheners are packed in aerosol form. Every aerosol consists of a gas-tight container, a valve closure, an actuator button and a protective cap. Some dispensers have dip tubes. The operation of an aerosol is based on the discharge of its contents by the pressure of a compressed gas or the vapour phase generated by a propellant present in the container as a liquid. The container may be constructed of glass, plastic, or metals like tinplate and aluminium.

Air freshener aerosols — Specification

## Scope

This Draft Zambian Standard specifies the requirements, method of test and sampling for air fresheners in aerosol form.

This Standard does not apply to products for which therapeutic claims are made.

## Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ZS 954  (all parts), Cosmetics and cosmetic products

ISO 22716, Cosmetics — Good Manufacturing Practices (GMP) — Guidelines on Good Manufacturing Practices

ISO 7010, Graphical symbols -- Safety colours and safety signs -- Registered safety signs

ISO 24153, Random sampling and randomization procedures

## Terms and definitions

No terms and definitions are listed in this document. ISO and IEC maintain terminological databases for use in standardization at the following addresses: — ISO Online browsing platform: available at http://www.iso.org/obp

## Requirements.

### General requirements.

        - All ingredients used including dyes, pigment and colours shall conform to all parts of EAS 377

        - The product shall contain acceptable amount of the ingredients necessary to effect the intended end use performance as stipulated on the label.

        - The manufacture shall certify that all the ingredients used such as mineral oils, extracts from natural herbs, dyes, perfumes etc. shall not be harmful or irritating to the skin or the sculp (where applicable).

        - The product shall not stain fabrics and general household furnishings.

        - The propellants used shall be hydrocarbons or any other propellants with low mammalian toxicity. Chloro-fluorocarbons (CFCs) banned under the Montreal Protocol for being ozone depleting substances shall not be used as aerosol propellants.

4.16      The flammability of an aerosol formulation shall be kept as low as possible to ensure safety during use.

4.1.8	The product shall be produced, prepared and handled in accordance with ISO 22716.

4.2	Specific requirements

The product shall also comply with the requirements given in Table 1 when tested in accordance with the method prescribed therein.

Table 1 — Specific requirements for air freshener aerosols

4.3	Heavy metals requirements

The product shall comply with the heavy metal requirements given in Table 2 when tested in accordance with the test methods specified therein

Table 2 — Heavy metal requirements for air freshener aerosols

4.1.7   The product shall not have any objectionable odour.

| Characteristic | Requirement | Method of test |
| --- | --- | --- |
| Delivery rate g/s, min, | 0.01 | Annex A |
| CFCs | Absent | Annex B |
| Net weight delivery m/m, %, min. | 95 | Annex C |
| Spray test | To pass test | Annex D |
| Valve leakage, g/year, max. | 5 | Annex E |
| General leakage | To pass test | Annex F |
| Stability of smell | To pass test | Annex G |
| Non-volatile matter, % m/m, min. | 10.0 | Annex H |
| Cloud point | To pass test | Annex J |
| pH | 3.0 – 8.5 | ZS 185-17 |

| Heavy metal | Limit mg/kg, max | Test method |
| --- | --- | --- |
| i) Lead (Pb) | 10.0 | ZS 185-16 |
| ii) Arsenic (As) | 2.0 |  |
| iii) Mercury (Hg) | 2.0 |  |
| Note- The total amount of heavy metals as lead, mercury and arsenic, in combination in the finished product shall not exceed 10 mg/kg |  |  |

## Containers

      - All filled aerosol containers shall be appropriately classified in terms of the flame propagation characteristics of their contents when determined in the manner described in Annex H.

- Highly flammable – If the average length of the flame is greater than 450 mm, or if the flame burns back to the actuator, or continues to burn when the test flame is extinguished.

- Flammable – If the average length of the flame is between 200 and 450 mm.

- Non-flammable – If the product does not flame or burn in the manner described above

      - The aerosol containers shall comply with the requirements of annex J with respect to internal pressure, materials of construction, filling volume, and performance

6	Packaging

The container shall be designed and manufactured to contain the product safely under normal conditions of distribution, storage and use.

- Labelling

  - The package shall be legibly and indelibly marked with the following information:

    - manufacturer’s name and address;

    - name of the product;

    - date of manufacture;

    - expiry date;

    - net content;

    - directions for use;

    - flammability class (word or symbol);

    - batch or code number;

    - country of origin;

    - The declaration ‘Contains no CFC's or ODS’;

    - All ingredients shall be declared in descending order of predominance.

The INCI label names shall be used. CFC – Chlorofluorocarbons

ODS – Ozone-depleting substances

INCI – International Nomenclature for Cosmetic Ingredients.

7.2	The following warnings shall be legibly and indelibly marked on the container:

- This container is pressurized’.

- Keep away from heat, naked flames and direct sunlight’.

- Do not puncture or incinerate even when empty’.

- Keep away from children’.

Any pictorial symbol(s) on the container shall be in accordance with ISO 7010.

## 8	Sampling

Representative samples shall be drawn for test from the market or anywhere else following in accordance with outlined ISO 24153. The samples shall be declared as conforming to the specification if they satisfy all the specified requirements.

# Annex A

(normative)

# Determination of delivery rate of dispenser

## Material and apparatus

#### Any suitable timing device

    - Balance having an accuracy to 0.01 g and with a capacity greater than 500 g

    - Pair of gloves made of cloth or fabric or towel for handling dispensers during test

    - Pair of tongs for removing dispensers from water bath

    - Water bath set at 26 °C ± 0.3 °C, thermostatically controlled

## Procedure

    - Hold a dispenser upright, spray for two seconds to fill the eduction tube. Then weigh the dispenser

    - Submerge the dispenser into the water bath for 15 min using tongs, remove the dispenser from the bath and immediately dry the container with a towel. Spray the dispenser in one continuous burst for 10 s. Re- weigh the dispenser.

    - Repeat the procedure and take an average of three tests. The difference between the maximum and minimum delivery rates shall not exceed 0.2 g/s.

## Calculation

The delivery rate, expressed in grams per second, shall be calculated as follows:

M 1  M 2

Delivery rate =

N

where

M1   is the initial weight, in grams, of the dispenser;

M2   is the final weight, in grams, of the dispenser; and

N	is the time in seconds.

# Annex B

(normative)

# Determination of propellant composition

  - Procedure

    - The analysis of the propellant mixture in most aerosol is carried out conveniently by gas chromatography. For sampling, a hypodermic needle is fitted to the valve of the aerosol can and approximately 0.5 g of the propellant is injected into the heavy duty centrifuge tube closed with a serum cap, containing about 8 ml of benzene. After mixing, 5-µl samples are taken out from this tube with a microlitre syringe and injected into the gas chromatography

    - Two 4 572 mm × 6.35 mm OD columns operated at 40 °C are recommended for the analysis containing 20 % weight hexadecane and diethylhexyl sebacate respectively on silanized chromosorb W60/S0 mesh.

The first column should be used mainly for initial screening and the second column for the confirmation and determination of the identified propellants

Table B.1 lists the relative retention data of the most widely used propellant together with some other fluorinate hydrocarbons and benzene used as the solvent in the two columns.

Table B.1 — Relative retention data of propellants

| Chemical name | Stationary phase diethylhexyl sebacate | Stationary phase hexadecane |
| --- | --- | --- |
| Octafluorocylobutane | 0.214 | 0.122 |
| 1-chloro-1,2,2-trifluoroethylene | 0.268 | 0.196 |
| Propane | 0.275 | 0.22 |
| 1,2-difluoroethane | 0.289 | 0.141 |
| Dichlorodifluoromethane | 0.296 | 0.220 |
| 1,2-dichloro-1,1.2,2-tetrafluoromethane | 0.345 | 0.290 |
| Isobutane | 0.366 | 0.378 |
| Monochlorodiflouoromethane | 0.368 | 0.152 |
| 1-chloro-1,1-difluoroethane | 0.402 | 0.236 |
| n- butane | 0.449 | 0.527 |
| Vinylchloride | 0.529 | 0.353 |
| Trichlorofluoroethane | 1.000 | 1.000 |
| 1,1,2-trichloro-1,2,2-tetrafluoroethane | 1.254 | 1.342 |
| Dichloromonofluoroethane | 1.354 | 0.515 |
| 1,2-dibromo-1,1,2,2-tetrafluoroethane | 1.634 | 1.363 |
| Methylene chloride | 2.565 | 1.070 |
| Benzene | 6.786 | 5.661 |

## B.2 Results

The sample shall be considered as having failed the test if it contains any of the CFCs in Table D.1.

# Annex C

(normative)

# Net weight delivery

  - For the determination of the net weight delivery, a random sample of at least three packages is selected. After the removal of any dust cover or caps not required for dispensing the product, the gross weight of each package is determined and after shaking for 15 s, the content of the lightest container is drained by holding the valve wide open. The exhausted container is weighed .The result is called wet-tare weight and is equal to the weight of the container plus any product remaining after draining.

  - Consequently, the regeneration allowance is determined and subtracted from the wet-tare weight to obtain the corrected wet-tare weight. The regeneration allowance is defined as the difference between the weight of the product which would be delivered through normal usage and the weight of the product delivered by the present accelerated procedure. It is calculated by multiplying the label weight of the container by 0.02 g and rounding the result to the next lowest gram.

  - By subtracting the corrected tare weight from the gross weight, the adjusted net weight of the package is obtained. If this is greater than 95 % of the label weight the lot is assumed to be satisfactory. However, if it is less than 95 % of the label weight, the lot is rejected

# Annex D

(normative)

Spray test

Procedure

Hand-shake the sample container for 15 sec. Spray for 10 seconds. The sample shall be deemed to have passed the test if defects such as “streamers” (solid or nearly solid stream) “droppers” (dripping valve) or no spraying are absent.

Annex E (normative)

Determination of valve leakage

Select 3 aerosol containers, and record the date and time to the nearest half-hour. Weigh each container to the nearest mg, and record the weight in mg of each, as W1. Place the containers in an upright position and retain at room temperature for not less than 3 days. Weigh each container again, recording the weight in mg of each, as W2 and recording the date and time to the nearest half hour. Determine the time, T, in hours, during which the containers were under test.

Calculate the leakage rate, in mg per year, of each container taken using the formula: (365) (24/T) (W1 - W 2)

Where plastic-coated glass aerosol containers are tested, dry the containers in a desiccator for 18 hours, and set upright in a constant-humidity environment for 24 hours prior to determining the initial weight as indicated above. Perform the test under the same humidity conditions. Empty the contents of each container tested by utilizing any safe technique (e.g. chill to reduce pressure). Empty the residual contents by first rinsing with suitable solvents, then with a few portions of methanol. Retain the container, the valve and all associated parts as a unit, heat as W3 and determine the net fill weight (W1 - W3) for each container tested.

If the average net fill weight has been determined previously, this value may be used as the net fill weight.

Annex F (Normative)

Testing of filled aerosol containers

F.2 Interpretation of results

Any filled aerosol container that shall leak, get distorted or burst as a result of this test shall be considered to have failed the test and shall be discarded.

  - Procedure

    - All filled aerosol containers shall be tested by immersion in a water bath set at 55 °C.

    - The container shall be such that the pressure generated within the immersed container reaches not less than 90 % of the pressure generated within the containers at equilibrium at 55 °C.

Annex G (Normative )

Stability of smell

G.3 Results

The product shall be taken to have passed the test if after 12 h, the smell of the sample can clearly be picked up.

G.2 Procedure

Put some pieces of bleached gauze which have been pre-washed in hot water without soap and dried into a porcelain cup. Spray continuously about 2 ml of the sample into this cup. After the gauze gets soaked, remove one piece using a pair of pincers. Without squeezing it, dry it in an environment with temperature of 27 °C ± 2

°C and humidity of 65 % ± 5 % for 12 h.

  - Apparatus

    - Porcelain cup

    - Pincers

    - Bleached gauze, 10 pieces, of dimension 5 cm x 10 cm

    - Thermometer

    - Hygrometer

Annex H (normative)

Flame propagation

H.1 Principle

The filled aerosol container is sprayed as a test flame under controlled conditions and the length of the burning spray cone is measured.

  - Apparatus

    - In its simplest form, the apparatus consists of a base marked at 0.15 m intervals, an adjustable stand to carry the aerosol container which may be raised or lowered to accommodate differences in container height, a means of measuring the burning spray cone (usually a one metre fuel placed horizontally at the same level as the top third of the flame, the hottest part) a means of igniting the spray cone in the form of a test flame 0.05 m ± 0.005 m in height (usually a candle flame is used) as shown in Figure H.1

    - Water bath maintained at 20 °C. This equipment shall be used to bring the aerosol container and its contents to equilibrium at 20 °C (heat the cans to 20 °C in the water bath)

450 mm

150 mm

Test specimen

#### Figure H.1 — Apparatus for flame propagation test

Adjustable stand

## Procedure

    - Place the aerosol container on the stand. Depress the actuator and adjust the height of the stand so that the spray cone will pass through the upper third test flame (hottest part).

    - Bring the aerosol container and its content to the equilibrium temperature of 20 °C. Place the container on the stand so that the point where the spray emerges is 0.15 m from the test flame. Then light the test flame and depress the actuator for 15 s to 20 s. Record the total length of the burning spray cone and specify whether or not it burns back to the actuator.

    - Extinguish the test flame and record whether the spray cone continues to burn while the actuator is depressed.

    - Repeat each test twice and record the flame length as the average of the three tests.

# Annex I

(normative)

# Aerosol containers

## Material

The container shall be made from any metal of uniform quality which can be shown to be suitable for the purpose. The aerosol dispenser shall be compatible with the intended contents and may be internally coated or otherwise surface treated with appropriate lacquers or chemical treatments to ensure compatibility.

## Construction

### General

The construction of the container shall be either seamless or with seams welded, soldered, brazed, double seamed or swaged.

### Soldered side seam containers

      - Soldered side seams shall not be used for group 2 containers where the pressure of the contents at 55 °C is greater than 8.3 x 105 N/m2.

      - If the pressure developed by the contents of the container is greater than 6.9 x 105N/m2 at 38 °C soldered side seams shall not be used for any container unless type tests have shown it to be satisfactory for the service intended.

## Internal finish

The aerosol dispenser may be internally coated or otherwise surface treated with appropriate lacquers or by chemical treatments. Internal treatments shall be compatible with the intended contents.

## Filling volume

    - The total liquid (together with any solid) contents at a temperature of 55 °C shall occupy not more than 90 % of the internal volume of the closed dispenser.

    - For containers having a concave end with a radius of curvature less than the diameter of the body and where the end reverses before the container bursts when tested in accordance with clause 6, the total liquid (together with any solid) contents at a temperature of 55 °C shall occupy not more than 95 % of the closed dispenser.

#### Performance requirements

All filled dispensers shall be tested in accordance with Annex F.

# Annex J

(normative)

# Determination of cloud temperature

J.2 Results

The product shall be taken to have passed the test if no turbidity appears at a temperature of 5 °C. The cologne should be transparent

  - Procedure

    - Pour 20 mL of cologne into a wide cylinder and close it with a plug. Insert a thermometer having scale up to –20 °C. Immerse the thermometer into the liquid in such a manner that its bulb is situated at the same distance from the bottom and walls.

    - Immerse the cylinder containing the liquid into a cooling mixture containing ice and salt. After cooling the sample to 5 °C, take out the cylinder, shake it and scan it in transmitted daylight, or in the light of a 40 w electric lamp.