|  | DZS 1333 FIRST EDITION |
| --- | --- |

Draft Zambian Standard

Household Fabric Softener — Specification

__________________________________________________________________

ZAMBIA BUREAU OF STANDARDS

AMENDMENTS ISSUED SINCE PUBLICATIONS

| Amdt No. | Date | Text affected |
| --- | --- | --- |

COPYRIGHT PROTECTED DOCUMENT

© ZABS 2017

All rights reserved. Unless otherwise specified, no part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from Zambia Bureau of Standards

DATE OF PUBLICATION

This Zambian Standard has been published under the authority of the Zambia Bureau of Standards in XXXX.

ZAMBIA BUREAU OF STANDARDS

The Zambia Bureau of Standards is the Statutory National Standards Body for Zambia established under an act of Parliament, the Standards Act, No. 4 of 2017 of the Laws of Zambia for the preparation and promulgation of Zambian standards..

REVISION OF ZAMBIAN STANDARD

Zambian Standards are revised, when necessary, by the issue of either amendments or revised editions. It is important that users of Zambian Standards ascertain that they are in possession of the latest amendments or editions.

CONTRACT REQUIREMENTS

A Zambian Standard does not purport to include all the necessary provisions of a contract. Users of Zambian Standards are responsible for their correct application.

TECHNICAL COMMITTEE RESPONSIBLE

The preparation of this Zambian Standard was entrusted upon the Soaps and Detergents Technical Committee (TC4/2) upon which the following organizations were represented:

Consumer Competition & Protection Commission (CCPC)

Food and Drug Control Laboratory

Tapera Industries

Zambia Bureau of Standards (ZABS)

Zambia Bureau of Standards	Email: zabs@zamnet.zm or infozabs@zamnet.zm

Lechwe House	Website: www.zabs.org.zm

Freedom Way South-end

P.O. Box 50259

Lusaka

FORWORD

This National Standard has been prepared in accordance with the procedures of ZABS. All users should ensure that they have the latest edition of this publication as normative documents are revised from time to time.

No liability shall attach to ZABS or its Director, employees, servants or agents including individual experts and members of its Technical Committees for any personal injury, property damage or other damages of any nature whatsoever, whether direct or indirect, or for costs (Including legal fees) and expenses arising out of the publication, use of, or reliance upon this ZABS publication or any other ZABS publication.

Compliance with a Zambian standard or other normative document does not of itself confer immunity from legal obligations

## Introduction

Household fabric softeners are stable aqueous dispersions of cationic materials used for providing fabric softening and static control benefits during laundering. Rinse-added fabric softening compositions contain, as the active component, di-hardened tallow dimethylammonium chloride and imidazolinium compounds substituted with two tallow groups. These materials are normally prepared in the form of a dispersion in water.

The dispersion of the base material contains other additives such as preservatives, colourant (dyes) and fragrances. Other fabric rinses contain additives to prevent or control foaming and electrolyte salts such as calcium chloride and thickener materials to control the viscosity. Other ingredients are added to modify the rheology and/or flow of the dispersion

Household fabric softeners — Specification

## Scope

This Zambian Standard prescribes the requirements, sampling and test methods for household fabric softeners.

### Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

ISO 862, Surface active agents — Vocabulary

ISO 2871-1, Surface active agents — Detergents — Determination of cationic active matter content — Part 1: High-molecular-mass cationic active matter

ISO 2871-2, Surface active agents — Detergents — Determination of cationic active matter content — Part 2: Cationic active matter of low molecular mass (between 200 and 500)

ISO 4316, Surface active agents — Determination of pH of aqueous solution — Potentiometric method

RS 403: 2019 Household fabric softener – Specification

## Terms and definitions

For the purposes of this document, the terms and definitions given in ISO 862 and the following apply.

ISO and IEC maintain terminological databases for use in standardization at the following addresses:

—	ISO Online browsing platform: available at http://www.iso.org/obp

### 3.1

fabric softener

laundry auxiliary product or laundry detergent ingredient that gives fabrics a soft feel, freshening, smooth surface and reduces static electricity

### 3.2

laundering

cleaning and restoring of textile materials to a serviceable condition using the washing and drying equipment commonly found in households

## Requirements

### General requirements

    - The household fabric softener shall have uniform dispersion with no lumps or phase separation, and shall not contain ingredients in quantities that are toxic to human beings.

    - The household fabric softener shall be uniform in colour with no un-dissolved or precipitated dyes that may stain the treated fabrics.

    - The household fabric softener, both as received and when dissolved in water, shall possess a pleasant, fresh odour and shall not develop an objectionable odour during storage at ambient temperature.

    - The household fabric softener shall remain stable for the declared shelf life without breaking the dispersion (phase separation), changes in colour and deterioration of the odour.

    - The household fabric softener shall disperse effectively or easily in standard hard water (total hardness150 ppm ± 20 ppm) at 30 °C.

    - The household fabric softener shall influence the properties of the fabric to improved wet-ability, softness and reduced static charge.

### Specific requirements

    - The household fabric softener shall comply with the specific requirements given in Table 1 when

tested in accordance with the test methods specified therein.

Table 1 — Specific requirements for household fabric softener

| Sl no. | Property | Requirement | Test method |
| --- | --- | --- | --- |
| i) | Cationic surfactant content, % w/w, min | 2.5 | ISO 2871 |
| ii) | pH, neat | 2.0 – 7.0 | ISO 4316 |
| iii) | Total viable counts, c.f.u/g, max. | 2 x 102 | ISO 21149 |
| v) | E. coli | Not detected | ISO 21150 |
| vi) | P. aeruginosa | Not detected | ISO 22717 |

## Packaging

The household fabric softener shall be packaged in suitable containers that withstand normal usage and transportation and that prevent leaking, drying out and contamination of the product.

## Labelling

Each container and bulk package shall be securely closed, legibly and indelibly labelled in English and/or any other official language (French, Kiswahili, etc) used in the importing East African Partner State with the following information:

- name of the product as "Fabric softener”

- manufacturer’s name and physical address;

- batch or code number;

- list of ingredients

- net content;

- country of origin;

- instructions for storage, use and disposal;

- date of manufacture”; and

- best before date.

- safety precautions/warnings

NOTE    The name, physical address of the distributor/supplier and trade mark may be added.

## Sampling

Sampling shall be done in accordance with Annex A.

## Annex A

(normative)

## Sampling

### Requirements

    - In drawing, preparing, storing and handling samples, the precautions in A.1.2 – A.1.7 shall be observed.

    - Samples shall not be taken from places exposed to damp air, dust or soot.

    - The sampling instruments shall be clean and dry when used.

    - The samples, the material being sampled, the sampling instruments and the containers for samples shall be protected from adventitious contamination.

    - The samples shall be placed in clean and dry glass containers. The sample containers shall be of such a size that they are almost completely filled by the sample.

    - Each container shall be sealed airtight after filling, and marked with full details of sampling which include, date of sampling, batch or lot number, name of manufacturer, and other important particulars of the consignment.

    - The samples shall be stored in such a manner that the temperature of the material does not vary unduly from the normal temperature, and that they are protected from light.

### Scale of sampling

    - In a single consignment, all the packages containing products of the same type and form, and drawn from the same batch of manufacture, shall constitute a lot. If the consignment consists of packages containing products of different types and forms, then the packages containing products of the same type, form and batch of manufacture shall be grouped together, and such group shall constitute a separate lot.

    - For ascertaining the conformity of the lot to the requirements prescribed in this standard, tests shall be carried out on each lot separately. The number (n) of packages to be selected for drawing the samples shall depend upon the size (N) of the lot and shall be in accordance with Table A.1.

#### Table A.1 — Scale of sampling

| No. of packages in the lot (N) | No. of packages to be selected (n) |
| --- | --- |
| 4 – 15 | 3 |
| 16 – 40 | 4 |
| 41 – 65 | 5 |
| 66 – 110 | 7 |
| 111 and above | 10 |
| NOTE When the size of the lot is three packages or less, the number of containers to be selected and the criteria for judging the conformity of the lot to the specifications should be as agreed on between the purchaser/inspector and the supplier. |  |

    - The packages shall be selected at random and to ensure randomness of selection, a random number table shall be used. In case such tables are not available, the procedure given below may be adopted.

Starting from any package, count all the packages in one order as 1, 2, 3,…, up to r and so on, where r is the integral part of N/n, (N being the lot size and n the number of packages to be selected). Every rth package thus counted shall be withdrawn to give a sample for the purposes of test.

### Preparation of gross samples, test sample and reference sample

    - Gross sample

From each one of the packages selected as in A.2, draw at random one or more containers. The material in the containers so chosen shall be nearly thrice the quantity required for purpose of test as indicated in A.4.

### Test sample

      - From the gross representing each form of synthetic detergent take a small but equal quantity of material and mix thoroughly into a composite sample which should be of a size sufficient to carry out triplicate testing for all the characteristics specified under A.4. The composite samples representing each form and type of the product shall be divided into three equal parts, one for the purchaser/inspector, another for the supplier, and the third for the referee.

      - The remaining portion of the material in each of the gross samples shall be divided into three equal pans, each forming an individual sample. One set of individual samples, representing the n selected packages shall be for the purchaser/inspector, another for the supplier, and the third for reference.

      - All the composite and individual samples shall be transferred to separate containers. These containers shall then be sealed airtight with stoppers, and labelled with full particulars of identification given in A.1.6.

### Reference samples

      - The reference samples shall consist of a composite sample and a set of individual samples. All the containers shall bear the seals of both the purchaser/inspector and the supplier, and shall be kept at a place agreed to between the two parties.

      - Reference samples shall be used in case of any dispute between the purchaser/inspector and the supplier.